X Station Preview mode (OS 3.0.03)

Preview mode allows you to view the current setting of any control without modifying it. This will work in both template mode and synth mode on the X-Station.

To activate preview mode simply hold down the ‘PLAY’ button for more half a second, to indicate the unit is in preview mode the ‘TEMPLATE COMMON’ LED will slowly flash.

Once the unit is in preview mode you can view the current setting of any control on the screen by moving it (or pressing it in the case of the buttons, touchpad or expression pedal).

To get out of preview mode simply press the play button again, or any of the other menu buttons (TEMPLATE EDIT, GLOBAL e.t.c).

Changes to synth ‘Compare’ function.

The ‘COMPARE’ button on the X Station now acts as a toggle switch to switch compare mode on and off.

When the unit is in compare mode the patch stored in the X Stations memory will be heard if you play the synth, and the unit will work as in preview mode as described above, displaying the settings set of the original patch.

WARNING: The ‘COMPARE’ button must be used to turn compare mode on and off, if you use the play button, the unit will come out of preview mode but still play the original patch.